General Requirements for Decks:

- Decks more than 30“ above finish grade are required to have a guardrail at least 36” (inches) in height.

- Stair railing, handrails, and landings are required with 4 or more risers.  Handrails need to be 34-38” measured vertically off the nosing of the treads.  Handrails shall have returns.

- Handrails cannot be less that 1 ¼“ or more than 2 “ if circular. If not circular, the perimeter dimension will have a min of 4” and a max of 6 ¼ “.

- No place in the deck’s guardrail will allow the passage of a 5” sphere.  Other than the triangular opening formed by the riser, tread and bottom rail of the guard shall allow a 6“ sphere.

- Deck risers can’t exceed 7 ¾“ and must not have more than 3/8” variance between any two risers.  Stair treads must be a minimum of 10”.  There may not be more than 3/8“ variance between tread depth.

- Deck Stairs are required to be a minimum of 36“ wide.  Where the stairs have two or more risers they will be served by a landing extending past the last tread nosing a minimum of 12” for the width of the stairs

- Supporting posts must rest on top and be connected to the footing.  Posts are not permitted to be buried in the ground or footing.

- Cantilevers for beams may not exceed 12”. Cantilevers for floor joist may not exceed 24”.

- Deck footing shall be dug to a depth of 36”.

- Flashing is required to separate the deck from the house.